EG6801 Week 4 Lab Worksheet

Data collection, task timing, and result interpretation using PID and MPC

# 1. Lab purpose

This lab turns your MATLAB plant plus Raspberry Pi controller setup into an evidence-driven embedded control experiment. You will test both PID and MPC under multiple scenarios, collect control and timing data, validate that the data is correct, and interpret the results using the task model T1-T5.

T1 Receive: get measurement y(t) from the plant

T2 Compute: run PID or MPC to compute command u(t)

T3 Send: transmit u(t) back to the plant

T4 Log: write cycle and task timing evidence

T5 Background stress: optional load used to test timing robustness

# 2. Learning outcomes

Define the minimum control and timing data that must be logged every cycle.

Run PID and MPC under different operating scenarios and compare behavior.

Check whether logged data is valid before drawing conclusions.

Measure timing metrics (latency, jitter, deadline misses) and relate them to task execution times.

Write evidence-based conclusions about control quality and real-time performance.

# 3. What you must submit

Task-structured controller code with T1-T5 functions (or equivalent named blocks).

A results folder containing separate runs for PID and MPC across the required scenarios.

cycle.csv for each run (signals and cycle timing).

tasks.csv for each run (per-task timings).

Two plots per controller at minimum: y vs r and loop latency over time.

A short interpretation summary answering the guided questions in Sections 11 and 12.

# 4. Lab at a glance

| Activity | Output |
| --- | --- |
| Setup checks and logging schema | Folder structure, columns defined |
| Instrument task timings T1-T5 | cycle.csv and tasks.csv logging works |
| PID scenarios (S1 and S2 required) | At least 2 PID runs saved |
| MPC scenarios (S1 and S2 required) | At least 2 MPC runs saved |
| Stress or robustness scenario (S3 required) | PID and MPC runs under S3 |
| Analysis and interpretation | Plots, metrics, answers to guided questions |

If time is tight, complete the required scenarios first (S1, S2, S3). Optional scenarios S4 and S5 are extensions.

# 5. Prerequisites and setup checklist

Tick each item before you begin:

[x] Lecture

[x] Set up Git repository

[x] Update MATLAB

[x] MATLAB/Simulink plant model runs correctly and sends and receives data.

[x] Raspberry Pi controller code runs for both PID and MPC modes.

[x] You know your control period dt (for example 20 ms for PID, larger if MPC compute is heavy).

[x] You can save CSV files on the Pi or host machine.

[x] Clock timestamps are available with enough resolution (milliseconds or microseconds).

# 6. Standard folder structure for this lab

Use one root folder and keep each run separate so your evidence is clean and comparable.

results/
  PID_S1_step_nominal/
    config.txt
    cycle.csv
    tasks.csv
  PID_S2_step_stress/
    config.txt
    cycle.csv
    tasks.csv
  PID_S3_disturbance_or_saturation/
  MPC_S1_step_nominal/
  MPC_S2_step_stress/
  MPC_S3_disturbance_or_saturation/

# 7. Logging schema (required columns)

You must use a consistent schema for every run. This is the backbone of later scheduler comparisons.

## 7.1 cycle.csv (one row per control cycle)

| Column | Description | Example |
| --- | --- | --- |
| cycle_id | Sequential cycle number | 0, 1, 2, ... |
| timestamp_start_ms | Cycle start timestamp | 105234.200 |
| timestamp_end_ms | Cycle end timestamp | 105254.440 |
| dt_target_ms | Target control period | 20 |
| loop_latency_ms | timestamp_end - timestamp_start | 20.240 |
| jitter_ms | Actual cycle interval - dt_target | 0.8 |
| deadline_ms | Cycle deadline (normally dt_target) | 20 |
| late_flag | 1 if loop_latency > deadline_ms else 0 | 0 or 1 |
| controller_mode | PID or MPC | PID |
| scenario_id | S1, S2, S3... | S1 |
| r | Setpoint | 10.0 |
| y | Measured output | 8.6 |
| e | Error = r - y | 1.4 |
| u | Control command | 0.72 |
| sat_flag | Actuator saturated? 0/1 | 0 |
| rx_seq | Received packet sequence | 542 |
| tx_seq | Sent packet sequence | 542 |

You may add more columns (for example PID terms or MPC solver info) but do not remove the required ones.

## 7.2 tasks.csv (one row per task execution)

| Column | Description | Example |
| --- | --- | --- |
| cycle_id | The cycle that this task belongs to | 42 |
| task_name | T1, T2, T3, T4, or T5 | T2 |
| task_start_ms | Task start timestamp | 111024.100 |
| task_end_ms | Task end timestamp | 111028.550 |
| exec_time_ms | task_end - task_start | 4.450 |
| deadline_ms | Task deadline if used (optional but recommended) | 10 |
| task_note | Optional context | MPC horizon=10 |

# 8. Task model and measurement points

Use this structure or an equivalent one. The key requirement is that each task is timed separately.

For each control cycle:
  cycle_start = now()
  T1 Receive: read y(t), rx_seq
  T2 Compute: run PID or MPC, produce u(t)
  T3 Send: send u(t), tx_seq
  T4 Log: write cycle row and task rows
  T5 Stress (scenario dependent): optional CPU or I/O load
  cycle_end = now()
  compute loop_latency, jitter, late_flag

Important: timestamp tasks around the real work. Do not place start and end timestamps in the wrong place or your timing data will look perfect but be false.

# 9. Scenario matrix (what to test and why)

You must test both controllers under multiple scenarios, not just one clean run. This reveals robustness, timing sensitivity, and trade-offs.

| Scenario | Required? | Description | What it reveals |
| --- | --- | --- | --- |
| S1 Step nominal | Yes | Step setpoint, no stress, no added disturbance | Baseline overshoot, settling, steady-state error |
| S2 Step with stress | Yes | Same as S1 but T5 stress ON | Timing robustness and deadline sensitivity |
| S3 Disturbance or saturation | Yes | Inject a disturbance or use actuator limits | Robustness, windup or constraint behavior |
| S4 Noise | Optional | Add sensor noise or measurement jitter | Sensitivity to noise and filtering |
| S5 Model mismatch | Optional | Change plant parameter(s) slightly | Generalization and controller robustness |

## 9.1 Recommended settings (use or adapt)

| Item | PID suggestion | MPC suggestion |
| --- | --- | --- |
| dt_target_ms | 20 to 50 ms | 50 to 100 ms (or slower if needed) |
| Run length | 15 to 30 s | 15 to 30 s |
| S1 setpoint | 0 to 1.0 step at t=2s | 0 to 1.0 step at t=2s |
| S2 stress | Enable T5 CPU loop or file burst | Enable same T5 stress |
| S3 disturbance | Pulse disturbance at t=8s | Pulse disturbance at t=8s |
| Actuator limit | u in [0, 1] or plant-safe limit | Same physical limit |

# 10. Worked examples for metrics (use these formulas)

## 10.1 Control metrics

Use simple, consistent metrics so you can compare PID and MPC fairly.

Overshoot (%) = ((y_peak - r_final) / r_final) * 100 for a positive step

Steady-state error (SSE) = |r_final - mean(y in final window)|

Settling time = first time after which y remains within a chosen band (for example +/- 2% of r_final)

Rise time = time from 10% to 90% of the final value (optional but useful)

Example (step to r_final = 10):
  y_peak = 12  -> Overshoot = ((12 - 10) / 10) * 100 = 20%
  In the last 2 seconds, mean(y) = 9.7  -> SSE = |10 - 9.7| = 0.3

## 10.2 Timing metrics

loop_latency_ms = timestamp_end_ms - timestamp_start_ms

Deadline miss if loop_latency_ms > deadline_ms

Miss rate = (number of late cycles / total cycles) * 100

Jitter = actual cycle interval - dt_target_ms (use absolute values if you want magnitude only)

Task bottleneck = task with highest mean or max exec_time_ms (often T2 or T4)

Example (dt_target = 20 ms):
  Cycle 41 latency = 23.4 ms -> late_flag = 1
  If 3 late cycles out of 500 -> miss rate = (3 / 500) * 100 = 0.6%

# 11. Step by step procedure

## 11.1 Part A - Setup and baseline checks

Create your results folder and the six run subfolders listed in Section 6.

Write a config.txt file template for each run with: controller mode, dt_target_ms, scenario id, run length, plant notes, and any gains or MPC parameters.

Confirm the plant communicates with the controller and that y(t) changes when you send a test command u(t).

Choose deadlines. For this week, set cycle deadline equal to dt_target_ms.

Choose the same physical limits for both PID and MPC so the comparison is fair.

Questions to answer during Part A

What is your dt_target_ms for PID and why?

What is your dt_target_ms for MPC and why?

What actuator limits are you using and why are they physically reasonable?

How will you keep the PID and MPC comparison fair?

## 11.2 Part B - Implement logging and task timing

Instrument your loop with cycle timestamps: timestamp_start_ms and timestamp_end_ms.

Add task timing around T1, T2, T3, T4, and T5 (when T5 is active).

Compute and log loop_latency_ms, deadline_ms, and late_flag for every cycle.

Log control signals r, y, e, and u every cycle. Compute e = r - y in code to avoid mistakes.

Add rx_seq and tx_seq if your communication layer supports sequence numbers.

Run a short 5 to 10 second test and open the CSV files to confirm rows are being written correctly.

Data correctness checks for Part B

Are timestamps monotonic (never going backward)?

Do cycle_id values increase by exactly 1 each cycle?

Do r, y, and u values stay within expected ranges?

Can you find any missing or blank values in the first 20 rows?

Does e equal r - y for checked rows?

## 11.3 Part C - PID experiments

Run the following scenarios using PID. Save each run in its own folder.

PID S1 Step nominal: Step setpoint (for example 0 to 1 at t=2 s), stress OFF. Run for 15 to 30 s and save cycle.csv and tasks.csv.

PID S2 Step with stress: Repeat S1 exactly, but turn T5 stress ON. Use the same step, same dt, same gains, and same run length.

PID S3 Disturbance or saturation: Choose one and document it in config.txt. Example A: inject a disturbance pulse at t=8 s. Example B: enforce actuator saturation so u hits a limit during the step.

For each PID run, plot y and r on the same graph and plot loop latency over time.

Record basic metrics: overshoot, settling time, SSE, mean latency, max latency, miss count, and miss rate.

PID interpretation questions

Compared with S1, what changed in S2 when T5 stress was enabled? Use timing evidence.

Did stress change only timing metrics, or did it also change control behavior (overshoot, SSE, settling)?

In S3, what evidence shows disturbance rejection or saturation behavior?

If u saturates, what happened to e over time and why?

Which PID task (T1, T2, T3, T4, or T5) was the timing bottleneck?

## 11.4 Part D - MPC experiments

Repeat the same scenarios with MPC so the comparison is fair.

MPC S1 Step nominal: Use the same plant and step scenario as PID S1. Keep physical limits the same.

MPC S2 Step with stress: Repeat S1 with T5 stress ON.

MPC S3 Disturbance or saturation: Repeat the same S3 type you used for PID.

Log the same required columns. If possible, add MPC-specific columns (for example solver_time_ms, iterations, constraints_active).

Plot y vs r and loop latency over time for all MPC runs.

Record the same metrics as PID so you can compare directly.

MPC interpretation questions

How did MPC control quality compare with PID in S1 (overshoot, SSE, settling)?

How did MPC timing compare with PID in S1 and S2 (mean and max latency, miss rate)?

Did stress affect MPC more than PID? What evidence supports your answer?

If you logged solver time, how does solver time relate to T2 exec_time_ms?

Was MPC still within deadline? If not, what parameter could you change first (for example horizon or dt)?

## 11.5 Part E - Validate the data before analysis

Do not compare controllers until your data passes sanity checks.

Open each cycle.csv and confirm required columns exist and values are populated.

Check 10 random rows manually: verify e = r - y and loop_latency_ms = timestamp_end_ms - timestamp_start_ms.

Check for impossible values (for example negative exec_time_ms, huge jumps in y, or repeated cycle_id rows).

Check sequence numbers (if used). Note any gaps and explain them (packet drop, restart, logging issue, etc.).

If data is wrong, fix the logging and rerun at least one short test before continuing.

Sanity-check questions

Did you find any data issue? If yes, what was it and how did you fix it?

Which single sanity check is most important in your experiment and why?

Can a controller look good with bad logs? Give one example from this lab.

## 11.6 Part F - Compare and interpret results

Now build an evidence-based comparison between PID and MPC.

Create a summary table with one row per run and columns for control and timing metrics.

Compare PID vs MPC in S1 first (no stress). Identify the baseline trade-off.

Compare S1 vs S2 for each controller to measure timing sensitivity to stress.

Compare S3 behavior to discuss robustness (disturbance rejection or saturation handling).

Use tasks.csv to identify which task dominates time in each controller (especially T2 and T4).

Write a short conclusion using numbers, not opinions.

Evidence-based conclusion guide

Which controller gave better control quality in the nominal case, and what metric proves it?

Which controller gave better timing reliability, and what metric proves it?

What is the main trade-off you observed (for example better control but slower compute)?

If you had to deploy on a tighter CPU budget, which controller would you choose and why?

What is one improvement you would test next week?

## 11.7 Part G - Final checkpoint and submission

[ ] Each required run folder contains config.txt, cycle.csv, and tasks.csv

[ ] At least one y vs r plot for PID and one for MPC

[ ] At least one latency plot for PID and one for MPC

[ ] Summary table completed

[ ] Guided questions answered in your notes or report

# 12. Result recording templates

## 12.1 Run summary table (fill in during the lab)

| Run ID | Controller | Scenario | Overshoot % | Settling s | SSE | Mean latency ms | Max latency ms | Miss rate % | Main bottleneck task |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PID_S1 | PID | S1 |  |  |  |  |  |  |  |
| PID_S2 | PID | S2 |  |  |  |  |  |  |  |
| PID_S3 | PID | S3 |  |  |  |  |  |  |  |
| MPC_S1 | MPC | S1 |  |  |  |  |  |  |  |
| MPC_S2 | MPC | S2 |  |  |  |  |  |  |  |
| MPC_S3 | MPC | S3 |  |  |  |  |  |  |  |

## 12.2 Data validity checklist (tick for each run)

| Run ID | No missing values | Time monotonic | e = r - y checked | No impossible values | Comments |
| --- | --- | --- | --- | --- | --- |
| PID_S1 |  |  |  |  |  |
| PID_S2 |  |  |  |  |  |
| PID_S3 |  |  |  |  |  |
| MPC_S1 |  |  |  |  |  |
| MPC_S2 |  |  |  |  |  |
| MPC_S3 |  |  |  |  |  |

## 12.3 Quick reflection (write before leaving)

Answer these in 1 to 2 sentences each.

Exit questions

What data was essential for control interpretation?

What data was essential for timing interpretation?

What did stress reveal that a clean run could not reveal?

How did you know your data was trustworthy?

What will you change next in your controller or task design?

# 13. Troubleshooting guide

| Symptom | Likely cause | What to check first |
| --- | --- | --- |
| y does not respond | No comms, wrong IP/port, or wrong packet format | Network settings, packet fields, rx and tx sequence |
| u stays constant | T2 not running or clamped | Task timings, sat_flag, controller mode |
| Large oscillation | Aggressive gains, dt mismatch, or noisy derivative | PID gains, dt_target, filter D term |
| MPC misses deadlines | Horizon too large or solver too slow | T2 exec_time_ms, solver time, reduce horizon or increase dt |
| Perfect timing under stress | Timestamping placed incorrectly | Move timestamps around the real work |
| CSV rows missing | Logging blocks loop or file I/O issue | T4 timing, buffered logging, file permissions |