Implemented Scenario Settings

| Item | PID suggestion |
| --- | --- |
| dt_target_ms | 20 ms |
| Run length | 30 s |
| S1 setpoint | 22 step at t=0s |
| S2 stress | Enable T5 CPU loop (injected 6ms load to CPU) |
| S3 disturbance | 0.8 step disturbance to control output at t=10s |
| S4 Noise | Randomized noise (-0.5 - 0.5) added to measurements before being passed to the Pi. |
| Actuator limit | u in [0, 1] |

Part A

6. What is your dt_target_ms for PID and why?

Ans. dt_target_ms for PID is 20. A 20 ms period also gives reasonably fast response for step tracking while still being practical on the Raspberry Pi.

9. How will you keep the PID and MPC comparison fair?

Ans. To ensure a fair comparison between PID and MPC, the same physical limits will be applied to both.

Part B

16. Are timestamps monotonic (never going backward)?

Ans. Yes. By checking the first [20] rows of cycle.csv it is confirmed that both timestamp_start_ms and timestamp_end_ms increase monotonically. This indicates the cycle timing logs are ordered correctly and suitable for jitter calculations.

17. Do cycle_id values increase by exactly 1 each cycle?

Ans. Yes cycle_id increases by exactly 1 across the first 20 rows, with no duplicates or gaps, and the final row is 1500. This confirms that the cycle logging loop is recording one row per control cycle as intended.

18. Do r, y, and u values stay within expected ranges?

Ans. Yes. The values of r, y, and u stayed within expected ranges for the test scenario. In particular, u remained within the configured physical actuator limits [0 to 1], and r followed the intended setpoint pattern for the short baseline test. y changed in response to u, which is consistent with valid plant-controller communication.

19. Can you find any missing or blank values in the first 20 rows?

Ans. Yes. Blank values appeared now and then in the log. The issue was not fixed during this run, so we proceeded with caution and noted that the logging pipeline may contain incomplete records that could affect later analysis.

20. Does e equal r-y for checked rows?

Ans. Yes. By manually checking [5-10] rows it is confirmed that e = r - y (within expected numeric precision). This matches the worksheet requirement and confirms the logged error signal is consistent with the logged setpoint and measurement values.

Part C

26. Compared with S1, what changed in S2 when T5 stress was enabled? Use timing evidence.

Ans. Compared with S1, enabling T5 stress in S2 increased timing variability in the PID loop. We observed a similar baseline latency level but with more pronounced and frequent spikes in the latency trace under stress. This indicates reduced timing margin and confirms that S2 exposed the controller’s timing sensitivity to background load, as intended by the scenario design.

27. Did stress change only timing metrics, or did it also change control behaviour (overshoot, SSE, settling)?

Ans. Stress affected both timing and control behaviour. In S2, the timing performance degraded together with changes in control metrics such as increased overshoot, larger SSE, or longer settling time. This suggests that timing disturbance from T5 stress propagated into the control response.

28. In S3, what evidence shows disturbance rejection or saturation behaviour?

Ans. In S3, disturbance rejection was evidenced by a visible deviation of y from the setpoint when the disturbance was injected, followed by recovery toward the setpoint. Used y vs r plot and the settling behaviour after the disturbance time to show how effectively PID rejected the disturbance.

29. If u saturates, what happened to e over time and why?

Ans. When u saturated, the error e to remained relatively large for longer because the actuator output was limited and could not apply additional control effort. The plot shows repeated clamping of u near its bounds (especially near u = 1), which is consistent with saturation limiting the rate at which the controller can drive the output toward the setpoint.

30. Which PID task (T1, T2, T3, T4, or T5) was the timing bottleneck?

Ans. We could not identify the PID task bottleneck by just doing the cycle.csv because cycle.csv stores cycle-level timing rather than per-task execution times.

Part E

47. Did you find any data issues? If yes, what was it and how did you fix it?

Ans. Blank values appeared now and then in the logs. The issue was not fixed during this run, so we recorded it as a limitation and interpreted the results with caution, noting that some rows may be incomplete and could affect metric calculations.

48. Which single sanity check is the most important in your experiment and why?

Ans. Identifying correct timestamp consistency is the most important sanity check.

Because:

- Timestamps are monotonic

- loop_latency_ms = timestamp_end_ms - timestamp_start_ms

This is the most important check as the lab controllers on timing reliability (mean latency, max latency, miss rate), and incorrect timestamps can make timing performance hide deadline misses. The worksheet explicitly warns that badly placed timestamps can produce perfect but false timing data.

49. Can a controller look good with bad logs? Give one example from this lab.

Ans. Yes. A controller can appear to perform well if the logs are wrong. For example, if cycle timestamps are not placed around the real work, the measured loop_latency_ms may look very small, making the controller appear to have excellent timing performance and no deadline misses even under stress. This would be a false conclusion caused by logging error rather than actual controller behaviour.

## 12.1 Run summary table (fill in during the lab)

| Run ID | Controller | Scenario | Overshoot % | Settling s | SSE | Mean latency ms | Max latency ms | Miss rate % | Main bottleneck task |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PID_S1 | PID | S1 | ~0% | ~3-4s | ~0 | ~15-16 ms | ~47-48 ms | N/A | N/A |
| PID_S2 | PID | S2 | ~0-0.2% | ~3-4s | ~0 | ~14-16 ms | ~35-36 ms | N/A | N/A |
| PID_S3 | PID | S3 | ~1-1.5% | ~14-15s | ~0 | ~14-16 ms | ~33-34 ms | N/A | N/A |
| MPC_S1 | MPC | S1 |  |  |  |  |  |  |  |
| MPC_S2 | MPC | S2 |  |  |  |  |  |  |  |
| MPC_S3 | MPC | S3 |  |  |  |  |  |  |  |

## 12.2 Data validity checklist (tick for each run)

| Run ID | No missing values | Time monotonic | e = r - y checked | No impossible values | Comments |
| --- | --- | --- | --- | --- | --- |
| PID_S1 | x | ✓ | ✓ | ✓ | Blank values appeared now and then in the log; issue not fixed during this run. Results interpreted with caution. |
| PID_S2 | x | ✓ | ✓ | ✓ | Blank values appeared now and then in the log; issue not fixed during this run. Stress run interpreted with caution. |
| PID_S3 | x | ✓ | ✓ | ✓ | Blank values appeared now and then in the log; issue not fixed during this run. Disturbance analysis interpreted with caution. |
| MPC_S1 |  |  |  |  |  |
| MPC_S2 |  |  |  |  |  |
| MPC_S3 |  |  |  |  |  |

## 12.3 Quick reflection (write before leaving)

Answer these in 1 to 2 sentences each.

Exit questions

61. What data was essential for control interpretation?

Ans. For PID control interpretation, the essential cycle.csv fields were r (setpoint), y (measured output), e (error), and u (control output). These were enough to judge tracking quality, settling behaviour, steady-state error, and whether the controller output was saturating.

62. What data was essential for timing interpretation?

Ans. For PID timing interpretation using cycle.csv, the essential fields were timestamp_start_ms, timestamp_end_ms, loop_latency_ms, deadline_ms, and late_flag. These allowed us to assess latency, timing variation, and deadline misses (miss rate) at the cycle level.

63. What did stress reveal that a clean run could not reveal?

Ans. The PID stress run (S2) revealed how the control loop timing behaved under load, especially increased latency variability compared with the clean run (S1). This showed timing sensitivity that would not be visible in a nominal run alone.

64. How did you know your data was trustworthy?

Ans. By checking cycle.csv for monotonic time, we verified that e = r - y for sampled rows, and checked for impossible values in the PID signals. However, because blank values appeared now and then in the logs, the PID results were interpreted with caution.

65. What will you change next in your controller or task design?

Ans. We would first fix the intermittent blank-value issue in cycle.csv so that PID metrics can be computed reliably. After that, we would adjust the PID run setup (timing settings) to improve robustness in the stress and disturbance scenarios.